Supplementary material

Estimating road traffic flow across Europe: application of improving air pollution modelling

Youchen Shen1*, Kees de Hoogh 2, 3, Oliver Schmitz4, John Gulliver5,6, Danielle Vienneau2,3, Roel Vermeulen1, Gerard Hoek1§, Derek Karssenberg4§

- Institute for Risk Assessment Sciences, Utrecht University, Utrecht, The Netherlands

- Swiss Tropical and Public Health Institute, Allschwil, Switzerland

- University of Basel, Basel, Switzerland

- Department of Physical Geography, Faculty of Geosciences, Utrecht University, Utrecht, the Netherlands

- Centre for Environmental Health and Sustainability, School of Geography, Geology and the Environment, University of Leicester, Leicester LE1 7HA, UK

- Population Health Research Institute, St George’s, University of London, UK

* corresponding co-author

§ shared last co-authors

S1	Supplementary Table	2

S2	Supplementary Figure	12

S3	Supplementary text	24

S3.1	Missing values in OSM data	24

S3.2	Road-specific models vs one-big model	24

# Supplementary Table

Table A1  Predictor variables used in Random Forests model.

| Data source | Resolution | Predictor | code | Buffer size (radius) or point estimate | Year of data collection |
| --- | --- | --- | --- | --- | --- |
| Road length from Open street maps (OSM) (OpenStreetMap contributors, 2017) | 25m | - Highway: Motorway, trunk, motorway link, trunk link - Primary: primary, primary link - Local: secondary, secondary link, tertiary, tertiary link - Residential - Unclassified | highway primary local residential unclassified | 50; 100; 200; 300; 400; 500m | April 2020 |
| Speed limit from OSM (OpenStreetMap contributors, 2017) | 5m | - Highway: Motorway, trunk, motorway link, trunk link - Primary: primary, primary link - Local: secondary, secondary link, tertiary, tertiary link - Residential - Unclassified | maxv_20_highway maxv_10_primary maxv_10_local maxv_10_ residential maxv_10_unclassified | 20m for highway; 10m for other road types | April 2020 |
| The number of lanes from OSM (OpenStreetMap contributors, 2017) | 5m | - Highway: Motorway, trunk, motorway link, trunk link - Primary: primary, primary link - Local: secondary, secondary link, tertiary, tertiary link - Residential - Unclassified | lane_20_highway lane_10_primary lane_10_local lane_10_residential lane_10_unclassified | 20m for highway; 10m for other road types | April 2020 |
| GEOSTAT population grid (EUROSTAT, 2015) | 1km | Population per grid cell | pop | 1; 5; 8; 10; 12; 15; 20; 25; 30; 45; 50; 75; 100; 150; 200 km | Static |
| Built-up type | 1km | A Boolean of 1 for areas with non-zero population, non-zero sum of impervious density within 500m circular buffer, and non-zero sum of residential areas within 500m circular buffer; otherwise, a Boolean of 0 | area_type | point | Static |
| ACEA report of vehicles in use in 2011 (ACEA, 2021) | 1km | Number of passenger cars Number of commercial vehicles | pcar2011 cveh2011 | 1; 5; 8; 10; 12; 15; 20; 25; 30; 45; 50; 75; 100; 150; 200 km | Static |
| Residential areas from CORINE land cover (v20) (CLC, 2018) | 100m | Number of cells of residential areas | res | 100; 200; 300; 400; 500; 600; 700; 800; 1000; 1200; 1500; 1800; 2000; 2500; 3500; 4000; 5000; 6000; 7000; 8000; 10000 m | 2012 |
| Impervious surface from Copernicus services (Copernicus, 2021) | 100m | Percentage of sealed area | imd | 100; 200; 300; 400; 500; 600; 700; 800; 1000; 1200; 1500; 1800; 2000; 2500; 3500; 4000; 5000; 6000; 7000; 8000; 10000 m | 2012 |
| Altitude 90m SRTM DTM v4 (Jarvis et al., 2008) | 90m | Altitude | elevation | point | Static |
| Slope (Jarvis et al., 2008) | 90m | Slope in degrees (0 slope<90), calculated from the altitude data | slope | point | Static |

Table A2 Proportion of road segments with missing information on speed limit from OpenStreetMap.

Table A3 Proportion of road segments with missing information on number of lanes from OpenStreetMap.

Table A4 Speed limit of passenger cars by country, road type, and area type

| Country | Urban highways | Rural highways | Urban major roads | Rural major roads | Residential roads |
| --- | --- | --- | --- | --- | --- |
| Albania | 80 | 110 | 40 | 80 | 30 |
| Andorra | 90 | 110 | 50 | 90 | 30 |
| Austria | 130 | 130 | 50 | 100 | 30 |
| Belgium | 120 | 120 | 50 | 80 | 20 |
| Bosnia and Herzegovina | 80 | 130 | 50 | 80 | 30 |
| Bulgaria | 120 | 140 | 50 | 90 | 30 |
| Croatia | 130 | 130 | 50 | 90 | 30 |
| Czechia | 80 | 130 | 50 | 90 | 30 |
| Denmark | 130 | 130 | 50 | 80 | 30 |
| Estonia | 100 | 100 | 50 | 90 | 30 |
| Faroes | 130 | 130 | 50 | 80 | 30 |
| Finland | 80 | 110 | 50 | 80 | 30 |
| France | 110 | 130 | 50 | 80 | 30 |
| Germany | 130 | 130 | 50 | 100 | 30 |
| Greece | 110 | 130 | 50 | 90 | 30 |
| Guernsey | 112 | 112 | 48 | 96 | 30 |
| Hungary | 110 | 130 | 50 | 90 | 30 |
| Iceland | 110 | 110 | 50 | 80 | 30 |
| Ireland | 120 | 120 | 50 | 90 | 30 |
| Isle of Man | 112 | 112 | 48 | 96 | 30 |
| Italy | 130 | 130 | 50 | 90 | 30 |
| Jersey | 112 | 112 | 48 | 96 | 30 |
| Latvia | 90 | 100 | 50 | 90 | 30 |
| Liechtenstein | 100 | 120 | 50 | 80 | 30 |
| Lithuania | 110 | 130 | 50 | 80 | 30 |
| Luxembourg | 110 | 130 | 50 | 90 | 30 |
| Monaco | 110 | 130 | 50 | 80 | 30 |
| Montenegro | 80 | 100 | 50 | 80 | 30 |
| Netherlands | 100 | 130 | 50 | 80 | 30 |
| North Macedonia | 80 | 130 | 50 | 80 | 30 |
| Norway | 100 | 100 | 50 | 80 | 30 |
| Poland | 100 | 130 | 50 | 90 | 20 |
| Portugal | 100 | 120 | 50 | 90 | 30 |
| Romania | 130 | 130 | 50 | 90 | 30 |
| San Marino | 130 | 130 | 50 | 90 | 30 |
| Serbia | 80 | 120 | 50 | 80 | 30 |
| Slovakia | 90 | 130 | 50 | 90 | 30 |
| Slovenia | 110 | 130 | 50 | 90 | 20 |
| Spain | 90 | 120 | 50 | 90 | 30 |
| Sweden | 110 | 110 | 50 | 70 | 30 |
| Switzerland | 100 | 120 | 50 | 80 | 30 |
| United Kingdom | 112 | 112 | 48 | 96 | 30 |
| Vatican City | 130 | 130 | 50 | 90 | 30 |

Table A5 Speed limit of heavy vehicles by country, road type, and area type.

| Country | Urban highways | Rural highways | Urban major roads | Rural major roads | Residential roads |
| --- | --- | --- | --- | --- | --- |
| Albania | 80 | 80 | 40 | 60 | 30 |
| Andorra | 90 | 110 | 40 | 90 | 30 |
| Austria | 80 | 80 | 50 | 70 | 30 |
| Belgium | 90 | 90 | 40 | 80 | 20 |
| Bosnia and Herzegovina | 80 | 130 | 50 | 80 | 30 |
| Bulgaria | 90 | 100 | 50 | 80 | 30 |
| Croatia | 90 | 90 | 50 | 80 | 30 |
| Czechia | 80 | 80 | 50 | 80 | 30 |
| Denmark | 80 | 80 | 50 | 80 | 30 |
| Estonia | 90 | 90 | 50 | 90 | 30 |
| Faroes | 80 | 80 | 50 | 80 | 30 |
| Finland | 80 | 80 | 50 | 80 | 30 |
| France | 80 | 90 | 50 | 70 | 30 |
| Germany | 80 | 80 | 50 | 70 | 30 |
| Greece | 80 | 85 | 50 | 80 | 30 |
| Guernsey | 96 | 112 | 48 | 80 | 30 |
| Hungary | 80 | 80 | 50 | 70 | 30 |
| Iceland | 110 | 110 | 50 | 80 | 30 |
| Ireland | 90 | 90 | 50 | 80 | 30 |
| Isle of Man | 96 | 112 | 48 | 80 | 30 |
| Italy | 80 | 100 | 50 | 70 | 30 |
| Jersey | 96 | 112 | 48 | 80 | 30 |
| Latvia | 80 | 80 | 50 | 80 | 30 |
| Liechtenstein | 80 | 80 | 50 | 80 | 30 |
| Lithuania | 90 | 90 | 50 | 80 | 30 |
| Luxembourg | 90 | 90 | 50 | 75 | 30 |
| Monaco | 80 | 90 | 50 | 70 | 30 |
| Montenegro | 80 | 100 | 50 | 80 | 30 |
| Netherlands | 80 | 80 | 50 | 80 | 30 |
| North Macedonia | 80 | 130 | 50 | 80 | 30 |
| Norway | 80 | 80 | 50 | 80 | 30 |
| Poland | 80 | 80 | 50 | 70 | 20 |
| Portugal | 80 | 90 | 50 | 80 | 30 |
| Romania | 110 | 110 | 50 | 80 | 30 |
| San Marino | 80 | 100 | 50 | 70 | 30 |
| Serbia | 90 | 90 | 50 | 70 | 30 |
| Slovakia | 90 | 90 | 50 | 90 | 30 |
| Slovenia | 80 | 90 | 50 | 80 | 20 |
| Spain | 90 | 90 | 50 | 80 | 30 |
| Sweden | 90 | 90 | 50 | 70 | 30 |
| Switzerland | 80 | 80 | 50 | 80 | 30 |
| United Kingdom | 96 | 112 | 48 | 80 | 30 |
| Vatican City | 80 | 100 | 50 | 70 | 30 |

Table A6 Speed limit of motorcycles by country, road type, and area type.

| Country | Urban highways | Rural highways | Urban major roads | Rural major roads | Residential roads |
| --- | --- | --- | --- | --- | --- |
| Albania | 110 | 130 | 50 | 90 | 30 |
| Andorra | 110 | 130 | 50 | 90 | 30 |
| Austria | 130 | 130 | 50 | 100 | 30 |
| Belgium | 120 | 120 | 40 | 80 | 20 |
| Bosnia and Herzegovina | 110 | 130 | 50 | 90 | 30 |
| Bulgaria | 95 | 95 | 50 | 80 | 30 |
| Croatia | 130 | 130 | 50 | 90 | 30 |
| Czechia | 80 | 120 | 50 | 90 | 30 |
| Denmark | 130 | 130 | 50 | 80 | 30 |
| Estonia | 100 | 100 | 50 | 90 | 30 |
| Faroes | 110 | 130 | 50 | 90 | 30 |
| Finland | 80 | 100 | 50 | 80 | 30 |
| France | 110 | 130 | 50 | 80 | 30 |
| Germany | 130 | 130 | 50 | 100 | 30 |
| Greece | 110 | 130 | 50 | 90 | 30 |
| Guernsey | 112 | 112 | 48 | 96 | 30 |
| Hungary | 110 | 130 | 50 | 90 | 30 |
| Iceland | 110 | 130 | 50 | 90 | 30 |
| Ireland | 120 | 120 | 50 | 90 | 30 |
| Isle of Man | 112 | 112 | 48 | 96 | 30 |
| Italy | 130 | 130 | 50 | 90 | 30 |
| Jersey | 112 | 112 | 48 | 96 | 30 |
| Latvia | 80 | 80 | 50 | 90 | 30 |
| Liechtenstein | 100 | 120 | 50 | 80 | 30 |
| Lithuania | 110 | 130 | 50 | 80 | 30 |
| Luxembourg | 110 | 130 | 50 | 90 | 30 |
| Monaco | 110 | 130 | 50 | 80 | 30 |
| Montenegro | 80 | 100 | 50 | 80 | 30 |
| Netherlands | 100 | 130 | 50 | 80 | 30 |
| North Macedonia | 80 | 130 | 50 | 80 | 30 |
| Norway | 100 | 100 | 50 | 80 | 30 |
| Poland | 100 | 130 | 50 | 90 | 20 |
| Portugal | 100 | 120 | 50 | 90 | 30 |
| Romania | 130 | 130 | 50 | 90 | 30 |
| San Marino | 130 | 130 | 50 | 90 | 30 |
| Serbia | 80 | 120 | 50 | 80 | 30 |
| Slovakia | 90 | 130 | 50 | 90 | 30 |
| Slovenia | 110 | 130 | 50 | 90 | 20 |
| Spain | 120 | 120 | 50 | 90 | 30 |
| Sweden | 110 | 110 | 50 | 70 | 30 |
| Switzerland | 100 | 120 | 50 | 80 | 30 |
| United Kingdom | 112 | 112 | 48 | 96 | 30 |
| Vatican City | 130 | 130 | 50 | 90 | 30 |

Table A7 Potential predictor variables used in air pollution models.

| Data source | Resolution | Predictor | code | Direction of effects (NO2/O3/PM2.5 and PM10) | Buffer size (radius in m) or point estimate | Year of data collection |
| --- | --- | --- | --- | --- | --- | --- |
| Open street maps | 25m | Major roads All roads | majorRoads allRoads | +/-/+ +/-/+ | 50; 100; 200; 300; 400; 500; 700; 1000; 2000; 5000; 10000 | April 2020 |
| Traffic estimates from RF models | 5m | Annual average daily traffic (AADT) estimates | AADT_onRoad | +/-/+ | point | 2019 |
| Traffic estimates from RF models | 5m | Annual average daily traffic (AADT) estimates in buffers | aadt | +/-/+ | 50; 100; 200; 300; 400; 500; 700; 1000; 2000; 5000 | 2019 |
| CORINE land cover (v20) (Copernicus, 2021a) | 100m | Number of cells of the land cover types: - Residential areas - Natural areas - Urban green - Industrial/commercial areas - Ports - Total built-up | res nat ugr ind por tbu | +/-/+ -/+/- -/+/- +/-/+ +/-/+ +/-/+ | 100; 200; 300; 400; 500; 600; 700; 800; 1000; 1200; 1500; 1800; 2000; 2500; 3500; 4000; 5000; 6000; 7000; 8000; 10000 | 2018 |
| Impervious surface from Copernicus services (Copernicus, 2021b) | 100m | Percentage of sealed area | imd | +/-/+ | 100; 200; 300; 400; 500; 600; 700; 800; 1000; 1200; 1500; 1800; 2000; 2500; 3500; 4000; 5000; 6000; 7000; 8000; 10000 | 2018 |
| GEOSTAT population grid (European Commission, 2011) | 1km | Population per grid cell | pop | +/-/+ | point | 2011 |
| Altitude 90 m SRTM DTM v4 (Jarvis, A., H.I. Reuter, A. Nelson, 2008) | 90m | Altitude | alt | -/+/- | point | Static |
| ERA 5 monthly aggregates from climate reanalysis (Copernicus, 2019; ECMWF, 2019) | 0.1o (~12km) | Annual averages from monthly aggregates: - Average temperature at 2m height (Kelvin) - Total precipitation (m) - Average surface pressure (Pa) - Average 10m eastward daily wind speed (m/s) - Average 10m northward daily wind speed (m/s) - Average 10m wind speed (m/s) | temp prep pres uwind vwind wind | +/+/+ -/+/- +/+/+ +/+/+ +/+/+ -/-/- | point | 2019 |
| PM2.5 derived from MODIS, MISR, and SeaWiFS satellite instruments | 0.01o (~1.2km) | Ground-level annual averaged PM2.5 concentrations derived from satellite | pm25_SAT | NA/NA/+ | point | 2018 |
| Aerosol Optical Depth from the MCD19A2 V6 product (LP DDAC, 2018) | 1km | Annual averages from the daily AOD values from MODIS Blue band B3 (0.47 µm) and Green band B4 (0.55 µm) over land | AOD047 AOD055 | NA/NA/+ | point | 2019 |
| Annual average NO2 derived from OMI (Boersma et al., 2011; TEMIS, 2020) | 0.25o (~13*24km) | Average tropospheric column density of NO2 | omi | +/NA/NA | point | 2019 |
| Geophysical-statistical estimates (Hammer et al., 2020; Van Donkelaar et al., 2019) | 0.01o (~1.2km) | Ground-level PM2.5 concentrations derived from satellite and being converted using GEOS-Chem CTM using GWR | gwr_sat | NA/NA/+ | point | 2019 |
| Annual average NO2, PM2.5 estimated by MACC-II Ensemble model (2010) (Marécal et al., 2015) | ~10km | Surface NO2, PM2.5 concentration estimated from an ensemble of chemical transport models | no2_10MACC pm25_10MACC | +/-/NA NA/NA/+ | point | Static (2010) |
| PM2.5, PM10, NO2, O3 (µg/m3) estimated by DEHM (Brandt et al., 2012; Christensen, 1997) | ~50km | Annual averaged concentrations (µg/m3) aggregated from monthly estimates of the chemical transport model, Danish Eulerian Hemispheric Model (DEHM) | NO2_dehm O3_dehm pm25_dehm pm10_dehm | +/-/NA NA/+/NA NA/NA/+ NA/NA/+ | point | 2019 |
| Climate zone id (Metzger, 2018) | 100m | Climatic biogeographical regions: Alpine, Atlantic, Continental, Northern, Southern (see Supplementary Figure 16) | zoneID | NA | point | Static (2018) |

# Supplementary Figure

Fig. A1 Locations of annual average daily traffic (AADT) observations.

Fig. A2 Scatterplots of traffic intensity observations in thousands (presented by AADT—annual average daily traffic intensity) collected in year 2013 (x-axis) and year 2013 (y-axis) in (a) Germany, and (b) the United Kingdom (UK). Observations in Germany were collected by automatic counters, whereas in the UK observations were collected by both automatic counters and manual counting. Blue lines represent regression lines, and black lines represent 1:1 lines.

Fig. A3 Boxplots combined with violin plots for observed AADT values (in thousand vehicles /day) across different road types and countries  (AT: Austria, CH: Switzerland, DE: Germany, FR: France, IT: Italy, UK: United Kingdom).

Fig. A4 Scatterplots between observed and estimated annual average daily traffic (AADT) counts (unit: thousand vehicles per day) for all road types. Best-fit linear line is in blue with fitted linear regression and adjusted coefficient of determination (R2) indicated in upper left corner, and 1:1 line is in black. Number of points (N) is also shown in upper left corner.

Fig. A5 Boxplots of values of top 15 most informative variables from residential RF model.

“cveh2011_1k”: number of commerical vehicles summed within 1km buffer; “cveh2011_200k”: number of commerical vehicles summed within 200km buffer; “elevation”: elevation; “local_300”: length of local roads summed within 300m buffer (i.e., secondary and tertiary roads defined in OpenStreetMap); “local_500”: length of local roads summed within 500m buffer (i.e., secondary and tertiary roads defined in OpenStreetMap); “pcar2011_200k”: number of passenger cars summed within 200km buffer; “residential_100”: length of residential roads summed within 100m buffer; “residential_200”: length of residential roads summed within 200m buffer; “residential_300”: length of residential roads summed within 300m buffer; “residential_400”: length of residential roads summed within 400m buffer; “residential_50”: length of residential roads summed within 50m buffer; “residential_500”: length of residential roads summed within 500m buffer; “slope”: slope in degrees (ranging from 0 to 90). More detailed descriptions can be found in Table A1.

Fig. A6 Boxplots of estimated AADT values in thousands (unit: thousand vehicles/day) across different road types (highway: motorway and trunk roads; primary roads; local: secondary and tertiary roads; residential roads as defined in OpenStreetMap).

Fig. A7 Scatterplots between national traffic intensity model estimates and Europe-wide random forest model estimates on annual average daily traffic (AADT) counts (unit: thousand vehicles per day) on a randomly selected subset of road segments in A) The Netherlands and B) Switzerland. Values were extracted on randomly selected road segments. Black lines are 1:1 line, and blue lines are regression lines.

Fig. A8 Bland-Altman plots between national traffic intensity model estimates and Europe-wide random forest model estimates on annual average daily traffic (AADT) counts (unit: thousand vehicles per day) separated by road type (highway, primary, local: secondary and tertiary, residential) in A) The Netherlands and B) Switzerland. Values were extracted on randomly selected centerline road segments (with AADT counts’ unit as thousand vehicles per day). Difference was calculated by subtracting national model estimates with EU-wide estimates. Red dashed lines give averages of differences between Swiss national traffic model estimates and Europe-wide model estimates, and black dotted lines give 95% limits of agreement of the differences (mean  1.96*standard deviation).

Fig. A9 Boxplots of natural logarithms of estimated AADT counts from national traffic models (national) and Europe-wide RF AADT models (eu) separated by road types and countries (A) The Netherlands, and B) Switzerland).

Fig. A10 Coefficient of variation for each centerline road segment used for extraction in a) the Netherlands and b) Switzerland separated by area type (urban and rural) and road type (highway, primary, local: secondary and tertiary, and residential).

Fig. A11 Scatterplots between sum of national traffic intensity model estimates and Europe-wide random forest model estimates for annual average daily traffic (AADT) (unit: 105 vehicles per day) within (a) 50m buffer, (b) 500m buffer, (c) 1km buffer, (d) 2km buffer, (e) 10km buffer separated by road types (with traffic intensity unit as thousand vehicles per day) for A) the Netherlands and B) Switzerlands. Values were extracted at 1795 randomly selected residential points across the Netherlands and 999 points across Switzerland. Black lines are 1:1 line, and blue lines are regression lines.

Fig. A12 Bland-Altman plots between sum of national traffic intensity model estimates and Europe-wide random forest model estimates on annual average daily traffic (AADT) (unit: thousand vehicles per day) within (a) 50m buffer, (b) 500m buffer, (c) 1km buffer, (d) 2km buffer, (e) 10km buffer separated by road types for A) the Netherlands and B) Switzerland. Values were extracted at 1795 randomly selected residential points across the Netherlands and 999 points across Switzerland. Difference was calculated by subtracting national model estimates with EU-wide estimates. Red dashed lines give averages of differences between Swiss national traffic model estimates and Europe-wide model estimates, and black dotted lines give 95% limits of agreement of the differences (mean  1.96*standard deviation)

Fig. A13 Top 10 informative vairables in air pollution RF models with and without traffic variables offered (i.e., aadt_onRoad and aadt_[buffer in m]).

.

Fig. A14 Five-fold cross-validated model performance of air pollution models with and without AADT estimates, evaluated by two metrics: (A) mean-square-error-based R2 (MSE-R2)and (B) root-mean-square-error (RMSE). Models were built for four pollutants: (A) NO2, O3, PM10 and PM2.5 for year 2019. Bars with dashed lines present models with AADT estimates and with solid grey fills present models without AADT estimates. Regression approaches include gwr (geographically weighted regression), rf (random forests), and slr (supervised linear regression). Text in white at the bottom of each bar shows the difference of metric values between models with and without AADT estimates.

Fig. A15 Five-fold cross-validated model performance of air pollution models with and without AADT estimates, evaluated by two metrics: (A) mean-square-error-based R2 (MSE-R2)and (B) root-mean-square-error (RMSE) at stations located in rural areas where population density is lower than 5000 km-2. Models were built for four pollutants: (A) NO2, O3, PM10 and PM2.5 for year 2019. Bars with dashed lines present models with AADT estimates and with solid grey fills present models without AADT estimates. Regression approaches include gwr (geographically weighted regression), rf (random forests), and slr (supervised linear regression). Text in white at the bottom of each bar shows the difference of metric values between models with and without AADT estimates.

Fig. A16 Contrasts in estimated conctrations, presented by 95th percentile minus 5th percentile, were calculated for each city, using all pixel values shown in Figure 7. Text in white at the bottom of each bar gives value of difference between 95th percentile and 5th percentile (unit: g/m3).

# Supplementary text

## Missing values in OSM data

We replaced missing values of number of lanes with the average values for different road types, which are 2 for highway, primary, secondary, tertiary roads and 1 for residential and unclassified roads. For speed limit, we replaced missing values with the national legal speed limit separated by road types: highway (motorway and trunk roads), non-highway (primary, secondary and tertiary), and residential roads (residential roads).

The national legal speed limit for highway and non-highway was further separated by area types: built-up and non built-up areas, determined by impervious data (Copernicus, 2021), residential areas of CORINE data (CLC, 2018) and population data (EUROSTAT, 2015). The national legal speed limit information was obtained from the European Commission and the Automoblie Club du Luxembourg (ACL, 2019; Commission, 2023) with missing values for non-residential roads assumed based on neighbouring countries and with missing values for residential roads assumed to be 30 km/hr. The national legal speed limit that we used is shown in Table A4-A6.

## Road-specific models vs one-big model

We trained the traffic models separately to estimate AADTs for four different road types (highway, primary, local, minor) with observation points categorized into these four road types based on the highest-ranking road type of the nearby roads. Based on our preliminary test, the reason of training the models separately was not because of a higher accuracy on model estimates, but rather because of the dominating highway-related variables in the variable importance of the one-big model and because of the low-to-medium correlations between the one-big model and the road-specific models.

We compared one big model (where only one model was trained to estimate AADTs for all road types) with the four separate models (where we also removed outliers of observations on residential roads with AADT higher than 5000 vehicles per day), based on their model performance (MSE-R2 and RMSE values using 5-fold CV), variables with high variable importance in RF and correlations of model estimates at 500 randomly-selected road segments across Europe. Although the model performance was similar between the one big model and the separate models for most road types (Fig. A17), the correlations of the model estimates at the 500 road segments were medium to low (0.59-0.85) for all road types except highway. In addition, the road-related variables with high variable importance in RF were dominated by highway-related variables, and no road-related variables with other road types (non-highway) were selected. Moreover, the RMSE of estimates on residential roads decreased from 3000 to 900 vehicles per day when we built a separate traffic model for residential roads (Fig. A17). The estimates of traffic intensity also on average became closer to the observed values for residential roads (Fig. A18).

Fig. A17 RMSE values of three different modelling settings (“one big model”: using all observations to build one model, “3 models”: using observations from 1) highway and trunk roads, 2) primary roads, and 3) secondary tertiary, and residential roads to build 3 separate models, “4 models”: using observations from 1) highway and trunk roads, 2) primary roads, 3) secondary and tertiary roads, 4) residential roads, and where the observations with AADT lower than 5000 vehicles per day were used on residential roads). R2 values of the final “4 models” setting are shown at the bottom.

Fig. A18 Boxplots of estimated values and observed values for annual average daily traffic (AADT) separated for road type and country (unit: vehicles per day). “rf-withRes” estimated AADT by modelling setting of “4 models”: using observations from 1) highway (motorway and trunk roads), 2) primary roads, 3) secondary and tertiary roads, 4) residential roads, and where the observations with AADT lower than 5000 vehicles per day were used on residential roads), “rf_noRes” estimated AADT by modelling setting of “3 models” (i.e., using observations from 1) highway (motorway and trunk roads), 2) primary roads, and 3) secondary, tertiary, and residential roads to build 3 separate models), and “obs” are values of AADT observations at the same estimation locations.

Passenger car is defined as “vehicles designed and constructed for the carriage of passengers and comprising no more than eight seats in addition to the driver’s seat, and having a maximum mass not exceeding 3.5 tons."

Commercial car is defined as "motor vehicles with at least four wheels used for the carriage of goods."

continuous urban fabric (high density housing) + discontinuous urban fabric (low density housing) (111-112)

Major roads include motorway, motorway_link, trunk, trunk_link, primary, primary link.

All roads include motorway, motorway_link, trunk, trunk_link, primary, primary link, secondary, secondary link, tertiary, tertiary link, residential.

Residential areas include continuous urban fabric (high density housing) and discontinuous urban fabric (low density housing) (codes 111-112)

Natural areas include forests and semi natural areas, wetlands, and water bodies (codes 311-523)

Urban green includes green urban areas and sport and leisure facilities (codes 141-142)

Industrial/commercial areas include Industrial or commercial units (code 121)

Ports include port areas (code 123)

Total built-up includes residential, Ind/comm, port, transport infrastructure, airports, mines, dumps and construction sites (codes 111-133)

For other years the buffered values were obtained by linearly interpolating the values from the closest two years)

For other years the buffered values were obtained by linearly interpolating the values from the closest two years)

‘NA’ indicates ‘not applicable, where the predictors were not used in the models for the pollutant.

The predictor zoneID was used as a spatial dummy variable.