Table of Contents

Appendix A. Supplementary data	3

Table A.1 Brief descriptions of the spatial variables.	3

Table A.2 GIS-derived land use potential predictors with pre-defined variable names, units, buffer sizes and direction of effect.	5

Table A.3 Selected variables in the annual LUR (CTM-incorporated) models.	7

Table A.4 Selected variables in the daily ST-LUR models.	7

Table A.5 Model performance of the annual LUR (CTM-incorporated) model and the differences with long-term ST-LUR models.	8

Figure A.1 Spatial distribution of monitoring sites by site type for NO2, PM2.5, PM10 and O3 in Great Britain across all study years	9

Figure A.2 Daily predicted vs daily measured concentrations of NO2	10

Figure A.3 Daily predicted vs daily measured concentrations of PM2.5	11

Figure A.4 Daily predicted vs daily measured concentrations of PM10	12

Figure A.5 Daily predicted vs daily measured concentrations of O3	13

Appendix B. Annual air pollution surfaces	14

Figure B.1 Estimated annual average NO2 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.	15

Figure B.2 Estimated annual average PM2.5 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.	16

Figure B.3 Estimated annual average PM10 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.	17

Figure B.4 Estimated annual average O3 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.	18

Figure B.5 Estimated annual average concentrations mapped at 25 x 25m resolution in London for 2015 for NO2, O3, PM2.5 and PM10 using the annual LUR (CTM-incorporated) models.	19

# Appendix A. Supplementary data

The following is the supplementary data to this article:

## Table A.1 Brief descriptions of the spatial variables.

| Data name | Yeara | Data source | Spatial coverage | Spatial resolution / geography | Temporal resolution | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Land cover | 2014 | Fecht, Beale and Briggs (2014) | GB | 25 m | - | CORINE-enhanced LCM2007 (combining CORINE Land Cover 2012 and two urban classes from Land Cover Map 2007). The available 31 land classes were aggregated into 9 main groups: high density urban, low density urban, other built-up, ports, industrial, natural areas, green space, agriculture, and watercourses. |
| Road network | 2016 | Ordnance Survey Meridian 2 | GB | line | - | Information on road network including motorway, A-road, B-road, and minor road |
| Traffic | 2016 | Morley and Gulliver (2016) | UK | line | - | Improved annual average daily traffic (AADT) (counts) of all roads, including improved AADT on minor roads using a routing algorithm |
| Topography | 2016 | Ordnance Survey Terrain 50 | GB | 50 m | - | Altitude |
| Building | 2019 | OS MasterMap Building Height Attribute | GB | polygon | - | Building footprint (area) and building height for all buildings. |
| Population and household | 2013 | Office for National Statistics (ONS) | UK | point | - | 2011 UK census. Containing population headcount and household count per postcode |

a Year/version of data when it was released

## Table A.2 GIS-derived land use potential predictors with pre-defined variable names, units, buffer sizes and direction of effect.

| Dataset | Predictor Variables | Variables Name | Unit | Buffer Size (m) | Direction of Effect |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | NO2 PM2.5 PM10 | O3 |
| Land cover | high density residential and commercial | HDURBAN | m2 | 100 200 300 400 500 1,000 3,000 5,000 | + | - |
|  | low density residential and commercial | LDURBAN | m2 |  | + | - |
|  | other built-up | OTHERBUILT | m2 |  | + | - |
|  | ports | PORTS | m2 |  | + | - |
|  | industrial | INDUSTRY | m2 |  | + | - |
|  | natural, semi-natural or forested areas | NATURAL | m2 |  | - | + |
|  | green space | GRESPACE | m2 |  | - | + |
|  | agriculture | AGRICULTURE | m2 |  | - | + |
|  | watercourses | WATER | m2 |  | - | + |
| Population/ Household | number of inhabitants in a buffer | POP | N |  | + | - |
|  | number of households in a buffer | HHOLD | N |  | + | - |
| Road Network | road length of all roads | ROADLEN | m | 25 50 100 150 200 300 400 500 1,000 | + | - |
|  | road length of A-roads | ROADLENA | m |  | + | - |
|  | road length of B-roads | ROADLENB | m |  | + | - |
|  | road length of minor roads | ROADLENMIN | m |  | + | - |
|  | road length of motorway | ROADLENMOT | m |  | + | - |
|  | road length of major roads (motorway and A-roads) | ROADLENMAJ | m |  | + | - |
| Traffic | total traffic load on all roads (ALL MV) | TRAFLOAD | veh/day*m |  | + | - |
|  | total traffic load on minor roads (ALLMV<10000) | TRAFLOADMIN | veh/day*m |  | + | - |
|  | total traffic load on major traffic roads (ALLMV>=10000) | TRAFLOADMAJ | veh/day*m |  | + | - |
|  | total heavy-duty traffic load on all roads (ALL C3) | HEAVYTRAFLOAD | veh/day*m |  | + | - |
|  | total heavy-duty traffic load on minor traffic roads (C3<275) | HEAVYTRAFLOADMIN | veh/day*m |  | + | - |
|  | total heavy-duty traffic load on major traffic roads (C3>=275) | HEAVYTRAFLOADMAJ | veh/day*m |  | + | - |
|  | traffic intensity on nearest road | TRAFNEAR | veh/day | N/A | + | - |
|  | traffic intensity on nearest major road | TRAFNEARMAJ | veh/day |  | + | - |
|  | heavy-duty traffic intensity on nearest road | HEAVYTRAFNEAR | veh/day |  | + | - |
|  | heavy duty traffic intensity on nearest major road | HEAVYTRAFNEARMAJ | veh/day |  | + | - |
|  | inverse distance to nearest road | INVDIST | m-1 |  | + | - |
|  | inverse distance to nearest road squared | INVDISTSQ | m-2 |  | + | - |
|  | inverse distance to nearest major road | INVDISTMAJ | m-1 |  | + | - |
|  | inverse distance to nearest major road squared | INVDISTMAJSQ | m-2 |  | + | - |
|  | traffic intensity on nearest road x inverse distance to the nearest road | INTINVDIST | veh/day/m |  | + | - |
|  | traffic intensity on nearest road x inverse distance to the nearest road squared | INTINVDISTSQ | veh/day/m2 |  | + | - |
|  | traffic intensity on nearest major road x inverse distance to the nearest major road | INTINVDISTMAJ | veh/day/m |  | + | - |
|  | traffic intensity on nearest major road x inverse distance to the nearest major road squared | INTINVDISTMAJSQ | veh/day/m2 |  | + | - |
|  | heavy-duty traffic intensity on nearest road x inverse distance to the nearest road | HEAVYINTINVDIST | veh/day/m |  | + | - |
|  | heavy-duty traffic intensity on nearest road x inverse distance to the nearest road squared | HEAVYINTINVDISTSQ | veh/day/m2 |  | + | - |
|  | heavy-duty traffic intensity on nearest major road x inverse distance to the nearest major road | HEAVYINTINVDISTMAJ | veh/day/m |  | + | - |
|  | heavy-duty traffic intensity on nearest major road x inverse distance to the nearest major road squared | HEAVYINTINVDISTMAJSQ | veh/day/m2 |  | + | - |
| Topography | altitude | ALTITUDE | m |  | + | - |
| Building | building volume within 50m of major road (sum of building area x height) in a circular buffer | BUILDINGVOL | m3 | 50 100 | + | - |

## Table A.3 Selected variables in the annual LUR (CTM-incorporated) models.

|  | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| NO2 | MACCa | TRAFLOAD_50 | ROADLEN_1000 | BUILDINGVOL_50 | INVDISTSQ |
| PM2.5 | MACCa | TRAFLOAD_50 | NATURAL_500 | BUILDINGVOL_50 |  |
| PM10 | MACCa | TRAFLOAD_50 | NATURAL_1000 | BUILDINGVOL_50 |  |
| O3 | MACCa | TRAFLOAD_50 | GRESPACE_300 | ROADLENMAJ_1000 |  |

a Annual average

## Table A.4 Selected variables in the daily ST-LUR models.

|  | Time-varying |  |  | Time-invariant |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 1 | 2 | 3 |
| NO2 | Meteorologya | MACCb |  | TRAFLOAD_50 | ROADLEN_1000 | BUILDINGVOL_50 |
| PM2.5 | Meteorologya | MACCb |  | TRAFLOAD_50 | NATURAL_1000 | BUILDINGVOL_50 |
| PM10 | Meteorologya | MACCb |  | TRAFLOAD_50 | NATURAL_1000 | BUILDINGVOL_100 |
| O3 | Meteorologya | MACCb |  | TRAFLOAD_50 | GRESPACE_300 | ROADLENMAJ_1000 |
| a Includes all four meteorology predictors: daily mean temperature, relative humidity, wind speed and wind direction. b Daily mean |  |  |  |  |  |  |

For NO2, the variable INVDISTSQ (inverse distance to nearest road squared) was not included in the final model because it did not improve the explained variance. Moreover, the buffer sizes of the selected variables for daily ST-LUR slightly differed from the annual LUR (CTM-incorporated) models. This is because the modified variables increased the explained variance in the daily measurements. For example, for PM2.5, NATURAL_1000 (natural land in a 1000 m buffer) was included in the final model instead of NATURAL_500 (natural land in a 500 m buffer).

## Table A.5 Model performance of the annual LUR (CTM-incorporated) model and the differences with long-term ST-LUR models. The differences for each indicator are recorded in the “diff.” columns.

| Pollutant | Year | N | CV-R2 | diff. | RMSE | diff. | RMSD | diff. | Bias | diff. | Slope | diff. | Intercept | diff. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NO2 | 2011 | 79 | 0.75 | 0.05 | 7.33 | -0.94 | 0.50 | 0.08 | 5.48 | -0.33 | 0.90 | 0.07 | 2.05 | -3.19 |
|  | 2012 | 73 | 0.75 | 0.09 | 7.40 | -1.52 | 0.46 | -0.04 | 5.47 | -0.79 | 0.95 | 0.13 | 0.74 | -4.80 |
|  | 2013 | 74 | 0.78 | 0.06 | 6.91 | -1.11 | 0.45 | -0.07 | 4.90 | -0.47 | 0.96 | 0.07 | 0.59 | -3.42 |
|  | 2014 | 74 | 0.77 | 0.06 | 7.17 | -0.88 | 0.41 | -0.04 | 4.99 | -0.60 | 0.93 | 0.04 | 1.11 | -2.44 |
|  | 2015 | 76 | 0.75 | 0.03 | 6.84 | -0.55 | 0.44 | -0.04 | 5.08 | -0.08 | 0.94 | 0.05 | 1.04 | -2.53 |
| PM2.5 | 2011 | 49 | 0.21 | -0.11 | 2.88 | -0.36 | 1.21 | 0.34 | 2.13 | -0.14 | 0.71 | 0.23 | 4.39 | -3.54 |
|  | 2012 | 54 | 0.30 | -0.08 | 2.40 | -0.37 | 1.09 | 0.22 | 1.74 | -0.19 | 0.82 | 0.31 | 2.27 | -4.26 |
|  | 2013 | 55 | 0.44 | -0.02 | 1.94 | -0.55 | 1.36 | 0.55 | 1.47 | -0.31 | 1.00 | 0.48 | 0.01 | -6.44 |
|  | 2014 | 52 | 0.48 | 0.03 | 1.56 | -0.61 | 1.15 | 0.19 | 1.18 | -0.39 | 0.89 | 0.40 | 1.40 | -4.99 |
|  | 2015 | 51 | 0.62 | 0.10 | 1.21 | -0.37 | 0.90 | 0.12 | 0.95 | -0.27 | 0.99 | 0.38 | 0.01 | -4.07 |
| PM10 | 2011 | 43 | 0.49 | -0.08 | 3.32 | 0.15 | 1.21 | 0.67 | 2.56 | 0.18 | 0.99 | 0.18 | 0.01 | -4.08 |
|  | 2012 | 47 | 0.53 | -0.05 | 2.80 | 0.06 | 0.95 | 0.14 | 2.26 | -0.03 | 0.96 | 0.13 | 0.68 | -2.73 |
|  | 2013 | 46 | 0.47 | -0.15 | 2.94 | 0.33 | 1.19 | 0.49 | 2.20 | 0.14 | 0.95 | 0.15 | 0.85 | -2.92 |
|  | 2014 | 39 | 0.59 | -0.06 | 2.26 | 0.01 | 1.00 | 0.23 | 1.62 | -0.25 | 0.82 | 0.08 | 3.30 | -1.22 |
|  | 2015 | 39 | 0.69 | 0.07 | 1.60 | -0.24 | 0.74 | 0.09 | 1.25 | -0.32 | 0.87 | 0.09 | 2.15 | -1.57 |
| O3 | 2011 | 55 | 0.74 | 0.08 | 4.84 | -0.79 | 0.44 | -0.03 | 3.86 | -0.43 | 0.96 | 0.07 | 1.89 | -3.54 |
|  | 2012 | 57 | 0.80 | 0.05 | 4.43 | -0.74 | 0.35 | -0.01 | 3.56 | -0.39 | 0.91 | 0.08 | 4.40 | -3.29 |
|  | 2013 | 55 | 0.91 | 0.03 | 3.13 | -0.68 | 0.26 | 0.00 | 2.50 | -0.55 | 1.00 | 0.10 | -0.03 | -5.03 |
|  | 2014 | 56 | 0.87 | 0.08 | 3.78 | -1.06 | 0.29 | -0.05 | 2.72 | -0.84 | 1.00 | 0.07 | 0.38 | -3.56 |
|  | 2015 | 55 | 0.86 | 0.08 | 3.74 | -1.15 | 0.34 | -0.07 | 2.99 | -0.75 | 1.01 | 0.13 | -0.13 | -6.46 |

CV-R2: cross-validated R2 of the annual LUR (CTM-incorporated) model
diff.: model performance parameter for annual LUR (CTM-incorporated) minus model performance parameter for long-term ST-LUR

## Figure A.1 Spatial distribution of monitoring sites by site type for NO2, PM2.5, PM10 and O3 in Great Britain across all study years. The four site types included in the NO2, PM2.5 and O3 models are background rural, background suburban, background urban and traffic urban. PM10 models do not have background suburban site.

## Figure A.2 Daily predicted vs daily measured concentrations of NO2. Sites are stratified by site type: beige – Background Rural, blue – Background Suburban, yellow – Background Urban, red – Traffic Urban.

## Figure A.3 Daily predicted vs daily measured concentrations of PM2.5. Sites are stratified by site type: beige – Background Rural, blue – Background Suburban, yellow – Background Urban, red – Traffic Urban.

## Figure A.4 Daily predicted vs daily measured concentrations of PM10. Sites are stratified by site type: beige – Background Rural, yellow – Background Urban, red – Traffic Urban.

## Figure A.5 Daily predicted vs daily measured concentrations of O3. Sites are stratified by site type: beige – Background Rural, blue – Background Suburban, yellow – Background Urban, red – Traffic Urban.

# Appendix B. Annual air pollution surfaces

Air pollution concentrations of points of interest (e.g. residential addresses) could be easily extracted from air pollution surfaces. The spatial resolution of the surfaces was set to 25 m. This was to 1) ensure the smallest buffer size (i.e. 25 m) of land use predictor variables was not aggregated during surface development, and 2) capture the concentration gradient of air pollution (especially NO2) around sources. However, we acknowledged that surface development at such a fine scale was computationally intensive. Specifically, Great Britain comprised 375,152,482 grid cells with a resolution of 25 m. Values of the selected predictor variables needed to be extracted at all grid cells, which involved computing relevant GIS techniques for each variable. Creating one such variable at 25 m resolution took 4 – 96 hours in ArcGIS on a standard computer (8Gb Random-access memory (RAM) Intel Core i7).

Once the predictor variables were compiled as individual raster layers, the annual surfaces can be easily and quickly developed from the intercept and coefficient values of the LUR or CTM-incorporated LUR models using the ‘raster calculator’ function in ArcGIS.

The maps below (Figure B.1 – B.5) show the annual air pollution surfaces of 2015 using the above CTM-incorporated LUR models. The maps are shown for Great Britain and regional (London) scale. The spatial resolution of the surfaces is 25 m. The surfaces are stored in raster format that can be directly used on various platforms, including ArcGIS and R.

## Figure B.1 Estimated annual average NO2 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.

## Figure B.2 Estimated annual average PM2.5 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.

## Figure B.3 Estimated annual average PM10 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.

## Figure B.4 Estimated annual average O3 concentrations mapped at 25 x 25m resolution in Great Britain for 2015 using the annual LUR (CTM-incorporated) model.

## Figure B.5 Estimated annual average concentrations mapped at 25 x 25m resolution in London for 2015 for NO2, O3, PM2.5 and PM10 using the annual LUR (CTM-incorporated) models.