| Environmental Sampling | qPCR |
| --- | --- |
| Up to 50 grams | Target genes (supplemental table 2) |
| Argentina, Bolivia, Brazil, Ecuador, Mexico, Paraguay, Peru | Hold stage 95°C, 20 sec; Amplification Denaturation 95°C, 1 sec; Annealing 60°C, 20 sec. |
| Table 1 | 2x TaqMan® Fast Advanced Master Mix (Applied Biosystems, Foster City, CA) 3.5 µl |
| Stored at 4°C and DNA extracted within 1 month | 2 µl of template |
| Exogenous DNA was used as an internal control to validate the extraction method. All samples had the internal control detect via qPCR | Primers were used at 900 nM (Thermofisher) Probe was used at 100 nM (Thermofisher) |
| Sample Treatment | QS7 Pro Fast Real-time PCR System (Applied Biosystems, Waltham, Massachusetts, USA) Or ABI 7500 Real-time PCR System (Applied Biosystems) Or Chia Portable Real-time PCR (Chia Bio, Santa Clara, CA) |
| Soil washed with PBS and 0.05% Tween 20 | 2 µl of PCR-water was used as a negative control |
| Flotation with 35.6% Sodium Nitrate solution or sugar solution (Specific Gravity 1.3) | Plasmids containing target parasite gene sequences was used as positive control |
| Sample Reduction | An exogenous DNA internal control was tested and all samples tested positive for the internal control Ct median 33.80 (31.94 - 37.08). |
| Samples are concentrated by a factor of 500 | Analysis - qPCR |
| Nucleic Acid Extraction | Positive control standard curves were performed in duplicate |
| MP fastDNA Spin kits for soil | Samples were tested in single |
| DNA eluent is 100 µl and stored at -20°C | All positive controls were compared to a set of known Ct values and were all within 5% range |
|  | Lowest standard measured was approximately 0.1 fg/µl per kg of soil |
|  | Automatic baseline and a threshold of 0.40 was used for all parasites |